Anexo 8.2 – SP 099 – Trecho de Planalto – 2034 – Dados de entrada e resultados de análise de nível de serviço (HCS 2000 – “TwoLane”)

HCS2000: Two-Lane Highways Release 4.1b

___________________Two-Way Two-Lane Highway Segment Analysis__________________

Analyst                 Vernon R. Kohl

Agency/Co.              CPEA / NOVOA

Date Performed          8/27/2009

Analysis Time Period    dia médio

Highway                 SP 099 - Rod. Tamoios

From/To                 Planalto

Jurisdiction            DER/SP

Analysis Year           2034

Description  EIA-RIMA PICP - Porto de São Sebastião

___________________________________Input Data_________________________________

Highway class  Class 2

Shoulder width       1.8     m      Peak-hour factor, PHF       0.88

Lane width           3.6     m      % Trucks and buses          35      %

Segment length       0.0     km     % Recreational vehicles     0       %

Terrain type         Rolling        % No-passing zones          100     %

Grade:  Length               km     Access points/km            1       /km

Up/down              %

Two-way hourly volume, V    2371    veh/h

Directional split       55  /   45  %

____________________________Average Travel Speed______________________________

Grade adjustment factor, fG                    0.99

PCE for trucks, ET                             1.5

PCE for RVs, ER                                1.1

Heavy-vehicle adjustment factor,               0.851

Two-way flow rate,(note-1) vp                  3198    pc/h

Highest directional split proportion (note-2)  1759    pc/h

Free-Flow Speed from Field Measurement:

Field measured speed, SFM                       -      km/h

Observed volume, Vf                             -      veh/h

Estimated Free-Flow Speed:

Base free-flow speed, BFFS                     80.0    km/h

Adj. for lane and shoulder width, fLS          0.0     km/h

Adj. for access points, fA                     0.7     km/h

Free-flow speed, FFS                           79.3    km/h

Adjustment for no-passing zones, fnp           1.1     km/h

Average travel speed, ATS                      38.3    km/h

__________________________Percent Time-Spent-Following________________________

Grade adjustment factor, fG                                  1.00

PCE for trucks, ET                                           1.0

PCE for RVs, ER                                              1.0

Heavy-vehicle adjustment factor, fHV                         1.000

Two-way flow rate,(note-1) vp                                2694    pc/h

Highest directional split proportion (note-2)                1482

Base percent time-spent-following, BPTSF                     90.6   %

Adj.for directional distribution and no-passing zones, fd/np 1.8

Percent time-spent-following, PTSF                           92.4   %

________________Level of Service and Other Performance Measures_______________

Level of service, LOS                                        F

Volume to capacity ratio, v/c                                1.00

Peak 15-min vehicle-kilometers of travel, VkmT15             0       veh-km

Peak-hour vehicle-kilometers of travel, VkmT60               0       veh-km

Peak 15-min total travel time, TT15                          0.0     veh-h